Feb. 9, 2003

Updated Feb. 21, 2003

Hi Woody,

Thanks for explaining the vocabulary requirements to me.  I’ll give a little back ground, and then try to specifically ask for what I think I need.

MDC has an ECG domain within it.  Under the ECG domain, there are several other categories:

- Lead names

- Beat names

- Wave component names

- Rhythm names

- Noise names

- Control variable names

- Contour observation names

- Maybe others…

In one of my Observation clones, I would like to restrict the “code” attribute to a new vocabulary domain called “ObservationSequenceTypes”.  Under that, I’ll need a couple of “time dimension” codes, and then an ECG child domain.  In the ECG child domain, I’ll reference the MDC ECG Lead Names (types).

In another Observation clone, I would like to restrict the “code” attribute to an ECG Annotation Type, which is really the list of categories above.  For the “value” attribute, I need to restrict the codes to one of the MDC codes for each of the types.

Below is the vocabulary hierarchy I envision.  I don’t know if has too many levels, or if it’s OK.  The reason I break out the ECG specific stuff as a child domain is because there may be other types of Observation Series and Annotations in the future (genomics, for example).  The domains in italics already exist.  Anything that starts with “e.g.” is just to give an example of what to expect from the external domain – it’s there just to enhance understanding.

- ActCode

  - (ObservationType)

    - (ObservationSeriesTypes)

      - (ECGObservationSeriesTypes)

        - ECG_RHYTHM_WAVEFORMS (This Observation Series type contains ECG “rhythm” waveforms.  The waveform samples are measured in absolute time (a.k.a. “subject time” or “effective time”).  These waveforms are usually “raw” with some minimal amount of noise reduction and baseline filtering applied.)

        - ECG_REPRESENTATIVE_BEAT_WAVEFORMS (This Observation Series type contains waveforms of a “representative beat” (a.k.a. “median beat” or “average beat”).  The waveform samples are measured in relative time, relative to the beginning of the beat as defined by the Observation Series effective time.  The waveforms are not directly acquired from the subject, but rather algorithmically derived from the “rhythm” waveforms.)

    - (ObservationSequenceTypes)

      - TIME_ABSOLUTE (A sequence of values in the “absolute” time domain.  This is the same time domain that all HL7 timestamps use.  It is time as measured by the Gregorian calendar.)

      - TIME_RELATIVE_TO_SERIES_EFFECTIVE_TIME (A sequence of values in a “relative” time domain.  The time is measured relative to the earliest effective time in the Observation Series containing this sequence).

      - (ECGObservationSequenceTypes)

        - (ECGLeadTypesMDC)

          - External vocabulary MDC, restricted to ECG Lead Types

            - e.g. MDC_ECG_LEAD_II

            - e.g. MDC_ECG_LEAD_V1

            - e.g. MDC_ECG_LEAD_AVR

    - (AnnotationTypes)

      - (ECGAnnotationTypes)

        - (ECGAnnotationTypesMDC)

          - External vocabulary MDC, restricted to ECG category names

            - e.g. MDC_ECG_BEAT

            - e.g. MDC_ECG_WAVC

            - e.g. MDC_ECG_RHY

            - e.g. MDC_ECG_NOISE

            - e.g. MDC_ECG_CONTOUR_OBS

  - (HL7DefinedActCodes)

    - (ActControlVariable)

      - (ECGControlVariables)

        - (ECGControlVariablesMDC)

          - External vocabulary MDC, restricted to ECG Control Variables

            - e.g. MDC_ECG_CV_LPFILTER

            - e.g. MDC_ECG_CV_HPFILTER

            - e.g. MDC_ECG_CV_ACFILTER

- (ObservationValue)

  - (AnnotationValues)

    - (ECGAnnotationValues)

      - (ECGBeatTypesMDC)

        - External vocabulary MDC, restricted to ECG Beat Types

          - e.g. MDC_ECG_BEAT_NORMAL

          - e.g. MDC_ECG_BEAT_SV_P_C

          - e.g. MDC_ECG_BEAT_V_P_C

      - (ECGWaveComponentTypesMDC)

        - External vocabulary MDC, restricted to ECG Waveform Component Types

          - e.g. MDC_ECG_WAVC_PWAVE

          - e.g. MDC_ECG_WAVC_TWAVE

          - e.g. MDC_ECG_WAVC_QRSWAVE

      - (ECGRhythmTypesMDC)

        - External vocabulary MDC, restricted to ECG Rhythm Types

          - e.g. MDC_ECG_RHY_SINUS

          - e.g. MDC_ECG_RHY_SINUS_TACHY

          - e.g. MDC_ECG_RHY_ATR_FLUT

      - (ECGNoiseTypesMDC)

        - External vocabulary MDC, restricted to ECG Noise Types

          - e.g. MDC_ECG_NOISE_CLEAN

          - e.g. MDC_ECG_NOISE_MODERATE

          - e.g. MDC_ECG_NOISE_SEVERE

      - (ECGContourObservationTypesMDC)

        - External vocabulary MDC, restricted to ECG Observation Types

          - e.g. MDC_ECG_CONTOUR_QT

          - e.g. MDC_ECG_CONTOUR_RR

          - e.g. MDC_ECG_CONTOUR_P_AXIS

Barry D. Brown 
Product Integration Manager 
Mortara Instrument, Inc. 
7865 N 86th St 
Milwaukee, WI 53224 
(414) 354-1600 x285 
mailto:Barry.Brown@mortara.com 
http://www.mortara.com